Creating Wildlife Ponds

A pond is basically a hole in the ground filled with water.  Small ponds can be lined with non-porous material so that they hold water.  This material could be concrete, rubber or thick black plastic.  If the pond is dug in clay soil that is non-porous there is no need for it to be lined.  Ponds are often built with stones and rocks.

The simplest way to make a pond is to buy an ‘instant pond’ made of fibre glass.  All you have to do is to dig a hole the right shape and drop the fibre glass shell into it.  The edge of the pond shell can hidden with a covering of concrete, paving stones or turf.

Another way of making a pond is to use a pond liner, a waterproof sheet made of rubber or thick black plastic.  A hole is dug to the required shape and size with sloping sides - it should be at least 1 to 1.5 metres deep.  The bottom of the pond should be built at different levels so that you can have a variety of plants.  Some water plants like shallow water and some prefer deep water.  The bottom of the hole must be covered with fine sand or with a layer of old newspaper, so that no sharp stones can pierce the liner.

After the liner has been laid over the hole the edges must be held down with paving stones, rockery stones or turf to keep it in place.  When the water is poured in the liner will mould itself to the shape of the hole.

- Once the pond is finished, whether it is clay, concrete, fibreglass or plastic lined, the bottom should be covered with a layer of soil for the plants to grow in.  It is best if you can add some mud taken from another existing pond.  A lot of small animals will be transferred to your pond in the mud.

- Plants can be bought from a water garden nursery or, with permission of the owner, you can collect plants from a local pond.  The plants should be dug into the soil at the depth they prefer.  Additional animals such as water beetles and pond snails may be also be found by pond dipping in an existing pond.

- For a wildlife pond you should ensure that only native species are planted in the pond.

- Especially avoid invasive weeds such as Canadian Pond Weed or Water Pennywort because they will grow quickly and smother other plant life. Some other ones that can cause problems are:

Parrot’s feather, NZ water stonecrop /Pygmy weed, Water fern and Nutall’s pondweed, Broad-leaved pondweed, Fennel pondweed, Bullrush (Reed Mace) and algae in shallow nutrient rich water.

Below is a list of plants and creatures that might inhabit a wildlife pond area:

Adult Damselfly

Adult Dragonfly

Adult Frog

Adult Mayfly

Adult Newt

Algae

Arrowhead

Caddisfly Larva

Cyclops

Damselfly Nymph

Dragonfly Nymph

Duck

Flatwork

Freshwater Shrimp

Tadpoles

Great Diving Beetle

Heron

Kestrel

Kingfisher

Leech

Marsh Marigold

Mayfly Nymph

Midge Larva

Newt Tadpole

Pond Skater

Pond Snail

Reed Mace

Water Boatman

Water Flea

Water Louse

Water Mite

Water Shrew

Water Worm